The current implementation of the SCU of the Basic Print Management Meta

SOP class follows the following algorithm.

Establish an association with a peer i.e. SCP

If association established then

Get Printer attributes

Create BASIC FILM SESSION SOP instance

For (each film of the film session)

Create a BASIC FILM BOX SOP instance

For (every image in the FILM BOX)

Set attributes of the BASIC IMAGE BOX SOP instance

Send action request

Delete the film box instance

Release Association

Description

-----------

This SCU implementation allows the users to provide the following

command line arguments.

Usage:

print_client [-a] [-c calledAPTitle] [-d] [-f films] [-i imageFormat]

[-p] [-t callingAPTitle] [-v] node port

Options:

a       Abort Association after echo requests

c       Called AP title to use during Association setup

d       Drop Association after echo requests

f       Number of films

i       Image display format

p       Dump service parameters after Association Request

t       Calling AP title to use during Association setup

v       Verbose mode for DUL facility

node    node name

port    port number

The SCU (client) starts with initializing the network and proposing the

role of an application entity requestor i.e. one who will request services.

Then the SCU requests service for the following two classes :

1) Basic Greyscale Print Management Meta SOP class

2) Basic Color Print Management Meta SOP class

Once these classes are negotiated, an association is established with the

peer by invoking the DUL routine viz. DUL_RequestAssociation.

If the initial phase of association establishment is successful, the

SCU proceeds to make use of the various DIMSE-N services useful for

this application.

A typical dump of the various parameters proposed in the association

look as follows :

Association accepted, parameters:

APP CTX NAME:1.2.840.10008.3.1.1.1

AP TITLE:    PRINT_CLIENT_SCU

AP TITLE:    PRINT_SERVER_SCP

AP TITLE:    PRINT_SERVER_SCP

MAX PDU:     16384

PRES ADDR:   dicom1

PRES ADDR:   dicom1:2700

REQ IMP UID: 1.2.840.113654.2.3.1993.1.1

REQ VERSION: MIRCTN20JUL93

ACC IMP UID: 1.2.840.113654.2.3.1993.1.1

ACC VERSION: MIRCTN20JUL93

Requested Presentation Ctx

Context ID:           1

Abstract Syntax:      1.2.840.10008.5.1.1.18

Result field:         0

Proposed Xfer Syntax(es)

1.2.840.10008.1.2

Accepted Xfer Syntax:

Context ID:           3

Abstract Syntax:      1.2.840.10008.5.1.1.9

Result field:         0

Proposed Xfer Syntax(es)

1.2.840.10008.1.2

Accepted Xfer Syntax:

Accepted Presentation Ctx

Context ID:           1

Abstract Syntax:      1.2.840.10008.5.1.1.18

Result field:         0

Proposed Xfer Syntax(es)

Accepted Xfer Syntax: 1.2.840.10008.1.2

Context ID:           3

Abstract Syntax:      1.2.840.10008.5.1.1.9

Result field:         0

Proposed Xfer Syntax(es)

Accepted Xfer Syntax: 1.2.840.10008.1.2

The SCU now sends an N-GET request to its peer (SCP) to get the

attributes of the PRINTER SOP instance which is maintained by the peer.

To send this N-GET request, the SCU is required to first build up the

N-GET request message. The request message is built up in the routine

"sendGetPrinterInstance". Whichever attributes the SCU wants are indicated

in the "attributeList" field of the request message. Once the request

message is built up, it is sent to the peer by invoking the SRV routine

viz. SRV_NGetRequest. This routine, on success, will provide a pointer

to the response message received from the SCP. The SCP then dumps the

various attributes received from the SCP as indicated in a typical

output below :

SCU : Sending N-GET Request to get printer attributes

SCU : N-GET response from the printer

Attributes of the printer are : -

DCM Dump Elements

Object type: ELEMENT LIST

Object size: 186

Group: 0008, Length:        0

0008 0000        4 //                ID Group Length//      22 34

0008 0070       10 //                ID Manufacturer//ABXY Inc.

0008 1090        8 //     ID Manufacturer Model Name//ABXY-LPF

Group: 0018, Length:        0

0018 0000        4 //               ACQ Group Length//      44 68

0018 1000       10 //       ACQ Device Serial Number//pq34mn.V8

0018 1002       12 //           ACQ Software Version//las-pri v3.2

0018 1200        8 //   ACQ Date of Last Calibration//19930721

0018 1201        6 //   ACQ Time of Last Calibration//141550

Group: 2110, Length:        0

2110 0000        4 //                               //Unimplemented

2110 0010        6 //          printer device status//NORMAL

2110 0020        8 //         additional information//NO-INFO

2110 0030       10 //                   printer name//XYZ Laser

DCM Dump Elements Complete

This actions are all taken in the "ngetCallback" routine.

The next step is to create an instance of the BASIC FILM SESSION SOP

class. The SCU uses the N-CREATE DIMSE service for this purpose.

TThis is handled by the routine "sendCreateFilmSession". In this

rotine the N-CREATE request message is built. The various mandatory

and optional attribute values are provided by the SCU in the request

message "dataSet" field. Since a data set is present, the SCU also

sets the dataSetType field to DCM_CMDDATAOTHER to indicate this.

In this implementation of the SCU, no facility is provided to dynamically

provide the attributes. Rather, all attributes are hard coded in the

form of a DCM_FLAGGED_ELEMENT map array. This array is converted into a

DCM_OBJECT i.e. dataSet using the DCM_ModifyElements routine.

Finally the request message is sent to the peer using the SRV routine

viz. SRV_NCreateRequest. On success, this SRV function provides

a pointer to the N-CREATE response message received from the SCP.

The SCU dumps the data set in the response message inside the

call back function - "ncreateBFSCallback".

Depending upon the command line argument value for number of films,

(default value = 1) the SCU then issues N-CREATE requests to create

instances of the BASIC FILM BOX SOP class. For each film box to be created,

the SCU proceeds in the same manner as described above. The N-CREATE

request message is built up in the "sendCreateFilmBox" routine and

the dumping of the data set received from peer takes in the

"ncreateBFBCallback" routine.

For every film box, once it is created, the SCU then proceeds to set

the attributes of the BASIC IMAGE BOX SOP instances which were created

as a result of creation of the FILM BOX instance. For this the N-SET

DIMSE service is used. The N-SET request message is built in the

"sendSetImageBox" routine. The data set returned by the SCP in the

response message is dumped in the "nsetBIBCallback" routine.

Now the SCU sends the N-ACTION request to the SCP requesting it to

print the images stored in the current FILM BOX instance. This

request message is built in the "sendActionRequest" routine and

the data set returned in the response message is dumped in the

"nactionBFBCallback" routine. This data set contains only the

instance uid of the PRINT JOB SOP class instance (if supported by

the SCP).

Finally, the SCU requests the SCP to delete the current FILM BOX

instance and the hierarchy rooted at that FILM BOX instance.

For this, the SCU builds up the N-DELETE request message (in the

routine "sendDeleteRequest") and upon receiving a response, verifies

if the status is a success or not (in routine "ndeleteCallback")

Status of current implementation

--------------------------------

The N-ACTION response does not contain the correct instance uid of

the PRINT JOB SOP instance since we are trying to figure out the

tag value for the actionReply field. So for now, to just test if the

code works, the SCP just sends some instance uid not encoded as an

ACTION REPLY.

The N-DELETE response will always be SUCCESS as of now since the SCP

code to handle delete requests is incomplete. So the SCP just sends

a SUCCESS code.

The N-EVENT-REPORT is not used by SCP/SCU as of now.